bcit

Lab 4 – ADC Interface

Nicholas Scott AKA “White Cheddar”

A01255181 | Feb. 10th, 2024

Contents

‘adcinterface.sv’	2

‘enc2chan’	5

‘lab4.sv’	6

RTL Netlists	8

Table of Figures

Figure 1 - 'adcinterface.sv' Simulation Waveforms	4

Figure 2 - 'adcinterface.sv' Simulation Transcript	5

Figure 3 - 'lab4.sv' Compilation Report	8

Figure 4 - 'adcinterface.sv' RTL Netlist	9

Figure 5 - 'lab4.sv' RTL Netlist	0

Figure 6 - 'enc2chan.sv' RTL Netlist	1

“Ninety-five percent of people who pass my test bench get it working the first time.”

- Sweet Bobby T

# ‘adcinterface.sv’

After much hardship, I settled on the following code for my ADC interface module:

// ELEX 7660 Lab 4

// Nicholas Scott AKA "White Cheddar"

// A01255181

// Feb. 4th, 2024

// Instructor: Sweet Bobby T

module adcinterface(

input logic clk, reset_n,   // clock and reset

input logic [2:0] chan,     // ADC channel to sample

output logic [11:0] result, // ADC result

// ltc2308 signals

output logic ADC_CONVST, ADC_SCK, ADC_SDI,

input logic ADC_SDO

);

logic [5:0] word; // word which tells ADC which channel we want

logic [3:0] count; // a count bit that counts

logic [11:0] tempResult; // a temp variable to store result

always_comb begin

// activate ADC clock for 12 clock cycles

ADC_SCK = ((count >= 2) && (count <= 13)) ? clk : 1'b0;

// Set config word based on chan input

case (chan)

0 : word = 6'b100010;

1 : word = 6'b110010;

2 : word = 6'b100110;

3 : word = 6'b110110;

4 : word = 6'b101010;

5 : word = 6'b111010;

6 : word = 6'b101110;

7 : word = 6'b111110;

endcase

end

always_ff @( negedge clk or negedge reset_n ) begin

if (~reset_n)

count <= 0;

else

count <= count + 1;  // automatically rolls over at 16

end

always_ff @( negedge clk or negedge reset_n ) begin

if (~reset_n) begin

ADC_CONVST <= 1;

ADC_SDI <= 0;

end

else begin

case (count)

0 : ADC_CONVST <= 0; // conversion start signal

// send config word one bit at a time

1 : ADC_SDI <= word[5];

2 : ADC_SDI <= word[4];

3 : ADC_SDI <= word[3];

4 : ADC_SDI <= word[2];

5 : ADC_SDI <= word[1];

6 : ADC_SDI <= word[0];

15 : ADC_CONVST <= 1; // conversion end

default : ADC_SDI <= 0;

endcase

end

end

always_ff @(posedge clk or negedge reset_n)

if(~reset_n)

result <= 0;

else

case (count)

// receive ADC result one bit at a time

2 : tempResult[11] <= ADC_SDO;

3 : tempResult[10] <= ADC_SDO;

4 : tempResult[9] <= ADC_SDO;

5 : tempResult[8] <= ADC_SDO;

6 : tempResult[7] <= ADC_SDO;

7 : tempResult[6] <= ADC_SDO;

8 : tempResult[5] <= ADC_SDO;

9 : tempResult[4] <= ADC_SDO;

10 : tempResult[3] <= ADC_SDO;

11 : tempResult[2] <= ADC_SDO;

12 : tempResult[1] <= ADC_SDO;

13 : tempResult[0] <= ADC_SDO;

14 : result <= tempResult; // assign result

default: result <= result;

endcase

endmodule

This code yielded the following simulation waveforms and transcript:

Figure 1 - 'adcinterface.sv' Simulation Waveforms

Figure 2 - 'adcinterface.sv' Simulation Transcript

# ‘enc2chan’

This code is basically the same as my ‘enc2freq.sv’ code, with the frequencies replaced with channels instead.

// ELEX 7660 Lab 4

// Nicholas Scott AKA "White Cheddar"

// A01255181

// Feb. 6th, 2024

// Instructor: Sweet Bobby T

module enc2chan

( input logic cw, ccw,       // outputs from lab 2 encoder module

output logic [2:0] chan,  // desired channel

input logic reset_n, clk); // reset and clock

logic [7:0] countup = 0;

logic [7:0] countdown = 0;

always_ff @(posedge clk) begin

if (reset_n) begin  // if active-low reset  is not pressed

if (cw)

countup <= countup + 1; // Count up for cw movement

else if (ccw)

countdown <= countdown + 1; // Count "down" for ccw movement

if (countup >= 4) begin // after 4 cw pulses

case (chan) // increment to next channel

0 : chan <= 1;

1 : chan <= 2;

2 : chan <= 3;

3 : chan <= 4;

4 : chan <= 5;

5 : chan <= 6;

6 : chan <= 7;

7 : chan <= 0; // cw rollover

default : chan <= 0;

endcase

countup <= 0;

end

else if (countdown >= 4) begin // after 4 ccw pulses

case (chan) // decrement to last channel

0 : chan <= 7;    // ccw rollover

7 : chan <= 6;

6 : chan <= 5;

5 : chan <= 4;

4 : chan <= 3;

3 : chan <= 2;

2 : chan <= 1;

1 : chan <= 0;

default : chan <= 0;

endcase

countdown <= 0;

end

end

else // set chan to zero if reset is pressed

chan <= 0;

end

endmodule

You didn’t ask us to write a test bench for this. Besides, I know it will work because ‘enc2freq.sv’ worked.

# ‘lab4.sv’

This module looks a lot like ‘lab3.sv’:

// ELEX 7660 Lab 4

// Nicholas Scott AKA "White Cheddar"

// A01255181

// Feb. 6th, 2024

// Instructor: Sweet Bobby T

module lab4 ( input logic ADC_SDO,        // ADC input

input logic CLOCK_50,       // 50 MHz clock

(* altera_attribute = "-name WEAK_PULL_UP_RESISTOR ON" *)

input logic enc1_a, enc1_b, // Encoder 1 pins

(* altera_attribute = "-name WEAK_PULL_UP_RESISTOR ON" *)

input logic enc2_a, enc2_b, // Encoder 2 pins

input logic s1, s2,         // reset and onOff pushbuttons

output logic [7:0] leds,    // 7-seg LED enables

output logic [3:0] ct,      // digit cathodes

output logic ADC_CONVST, ADC_SCK, ADC_SDI // ADC outputs

);

logic [1:0] digit;  // select digit to display

logic [3:0] disp_digit;  // current digit of count to display

logic [15:0] clk_div_count; // count used to divide clock

logic [2:0] chan;  // channel

logic [11:0] result; // ADC result

logic [7:0] enc1_count, enc2_count; // count used to track encoder movement and to display

logic enc1_cw, enc1_ccw, enc2_cw, enc2_ccw;  // encoder module outputs

// instantiate modules to implement design

decode2 decode2_0 (.digit,.ct) ;

decode7 decode7_0 (.num(disp_digit),.leds) ;

encoder encoder_1 (.clk(CLOCK_50), .a(enc1_a), .b(enc1_b), .cw(enc1_cw), .ccw(enc1_ccw));

enc2chan enc2chan_1 (.clk(CLOCK_50), .cw(enc1_cw), .ccw(enc1_ccw), .chan(chan), .reset_n(s1));

adcinterface adcinterface_1 (.clk(clk_div_count[15]), .reset_n(s1), .chan(chan), .result, .ADC_CONVST, .ADC_SCK, .ADC_SDI, .ADC_SDO);

// use count to divide clock and generate a 2 bit digit counter to determine which digit to display

always_ff @(posedge CLOCK_50)

clk_div_count <= clk_div_count + 1'b1 ;

// assign the top two bits of count to select digit to display

assign digit = clk_div_count[15:14];

// Select digit to display (disp_digit)

// the right three digits display the ADC output in hexadecimal

// the leftmost digit displays the channel

always_comb begin

case (digit)

0 : disp_digit = result[3:0]; // result LSN (Least Significant Nibble)

1 : disp_digit = result[7:4]; // the middle nibble

2 : disp_digit = result[11:8]; // result MSN

3 : disp_digit = chan; // Set digit 3 channel

endcase

end

endmodule

This code yielded the following compilation report:

Figure 3 - 'lab4.sv' Compilation Report

# My Issue

The reason I couldn’t demo on the first week is because I was defining my ‘word’ variable incorrectly. For some reason, ModelSim did not complain about it, but Quartus didn’t like it. I found out that:

The code on the left worked in ModelSim, but not Quartus. I suspect that ModelSim was smart enough to realize that I was concatenating single-bit logic, but Quartus assumed a width of 32 bits for each of the values, and thus the leading 0’s caused ‘word[5:0]’ to be all 0’s, and hence I was permanently stuck on channel 0 and 1 differential mode. Very silly of me.

# RTL Netlists

This whole design yielded the following RTL Netlists:

Figure 4 - 'adcinterface.sv' RTL Netlist

Figure 5 - 'lab4.sv' RTL Netlist

Figure 6 - 'enc2chan.sv' RTL Netlist